Problem Understanding

1. What exactly is the problem you are solving?
We are developing an AI-powered system to automatically detect man-made marine debris and underwater anomalies from Side-Scan Sonar imagery and convert detections into geospatial information.

2. Why is Side-Scan Sonar (SSS) used instead of normal underwater cameras?
SSS works in dark underwater environments and produces acoustic maps of the seabed, whereas cameras depend on light and water visibility.

3. How does Side-Scan Sonar work?
SSS sends acoustic signals toward the seabed and uses the reflected signals to create an acoustic image of underwater objects and terrain.

4. What kind of objects can be detected using SSS?
Our system targets ghost nets, shipwrecks, pipes, tyres, plastic waste and other man-made underwater anomalies.

5. What makes marine debris difficult to detect?
Speckle noise, low contrast, acoustic shadows, natural seabed patterns and AUV motion distortion make automatic detection difficult.

6. How do you differentiate marine debris from natural seabed objects?
We combine AI-based detection with acoustic-shadow verification to distinguish man-made targets from natural seabed features.

7. What is an acoustic shadow?
An acoustic shadow is the dark region behind an object where the sonar signal is blocked or reduced by the object.

8. Why is an acoustic shadow useful for detection?
A bright reflection followed by a corresponding dark shadow provides physical evidence that an elevated object exists on the seabed.

9. What are the major sources of noise in sonar images?
Major issues include speckle noise, acoustic signal distortion, natural seabed patterns and AUV motion-related distortion.

10. What happens when debris and the seabed have similar acoustic signatures?
The model may produce false positives, so we use sonar-specific preprocessing and acoustic-shadow verification to improve discrimination.

AI Model Questions

11. Which AI model are you using and why?
We use YOLOv8-OBB because it provides fast single-stage detection and handles arbitrarily oriented sonar targets using rotated bounding boxes.

12. Why did you choose YOLO instead of Faster R-CNN or U-Net?
YOLO provides faster real-time detection for edge deployment, while U-Net is mainly suited for pixel-level segmentation and Faster R-CNN is comparatively computationally heavier.

13. What are IoU, precision, recall and F1-score?
IoU measures box overlap, precision measures correct predictions, recall measures detected actual targets, and F1-score balances precision and recall.

14. Why not use a transformer-based model?
Our priority is lightweight, low-latency edge inference, so YOLOv8-OBB is more suitable for the intended AUV deployment.

15. How will you improve the model's accuracy?
We can improve accuracy using more representative sonar datasets, better preprocessing, augmentation, hyperparameter tuning and acoustic-shadow verification.

Sonar Image Processing

16. What preprocessing are you applying to sonar images?
We apply Lee/Frost denoising, CLAHE, normalization and slant-range correction before AI inference.

17. Why do you need preprocessing?
Preprocessing reduces sonar noise and improves contrast and image quality so the detection model can extract target features more effectively.

18. What is a Lee filter?
A Lee filter is an adaptive despeckling filter used to reduce acoustic speckle noise while preserving important image structures.

19. Why would you use a median/Gaussian filter?
They can smooth unwanted noise and improve image quality, although our documented pipeline specifically proposes Lee/Frost filtering.

20. How does sonar speckle noise affect detection?
Speckle noise creates grainy and distorted patterns that can hide target features and increase false detections.

Datasets

21. Which dataset are you using?
Our primary benchmark is the UATD (Underwater Acoustic Target Detection) Dataset, containing approximately 9,200 acoustic frames.

22. What is UATD?
UATD stands for Underwater Acoustic Target Detection Dataset, used as our primary benchmark for underwater acoustic target detection.

23. What is SCTD?
The provided project documentation does not specify the full form or details of SCTD, so we should not claim an expansion without verifying the exact dataset.

24. What is the SSS Crab Pot Detection dataset?
It is a Side-Scan Sonar dataset focused on detecting crab pots and provides additional sonar data relevant to man-made underwater-object detection.

25. What classes are present?
Our current class mapping contains ball, cube, human body, tyre, plane, ROV, square cage, circle cage, cylinder and metal bucket.

26. What are range, azimuth, elevation, soundspeed and frequency?
Range is target distance, azimuth is horizontal direction, elevation is vertical angle, soundspeed is the speed of sound in water, and frequency is the acoustic signal's cycles per second.

GPS + Sonar Metadata

27. How will you determine the location of detected debris?
We combine the detection's image position with sonar range/azimuth and vehicle survey-position/heading metadata through coordinate transformation to obtain latitude and longitude.

28. How is GPS obtained underwater?
GPS itself does not work effectively underwater, so underwater positioning requires systems such as INS, DVL or acoustic positioning, with GPS fixes typically available when the vehicle reaches the surface.

29. How are sonar range and azimuth used?
Range gives the target's distance and azimuth gives its direction, which are combined with the vehicle position and heading for geolocation.

Edge Computing

30. What is ONNX?
ONNX (Open Neural Network Exchange) is a model format that allows the trained YOLO model to be exported and optimized for deployment using runtimes such as ONNX Runtime or TensorRT.

Training & Testing

31. How are you evaluating your model?
We evaluate using Precision, Recall, mAP@50 and mAP@50–95, with the documented final results of 97.30% precision, 96.74% recall, 96.97% peak mAP@50 and 58.78% peak mAP@50–95.

32. What is your train/validation/test split?
The documentation identifies UATD_Training, UATD_Test_1 and UATD_Test_2, but the exact train/validation/test split is not yet documented and should be finalized before presenting it as a final evaluation protocol.